Translated from Chinese (Simplified) to English - www.onlinedoctranslator.com

FMC006GE31-Modbus address definition

The FMC006GE31 controller uses RS232 (DB9 female) or RS485 (Phoenix terminal) interface for Modbus communication. RS485 can support 120R resistor matching. The communication protocol uses Modbus RTU protocol, and the check format is CRC16. The function codes supported by this protocol are: 0X03, 0X05, 0X06, X10. The parameters of this controller are composed of two registers, which are 32-bit data, and the format is: (address high << 16 | address low). A single register is 16 bits. This controller has two data types: relay and register. Relay has only two types of values: 0x0000, 0xFF00. The RS485 communication cycle of this controller is 20ms. If it is lower than this time interval, communication failure will occur.

1. 0x03 function code
This function code is used to read the value of the controller's internal register. At least one register value is read each time.
Example of use:
Refer to the address table to read the baud rate of RS232 in slave address 01. The starting address is 01 14, which occupies two registers. The instructions are as follows:
send:
01 03 01 14 00 02 85 F3
Slave address function code read register start address read number check code
take over:
01 03 04 00 01 C2 00 FA 93
The number of bytes read from the slave address function code Address 0114 Data check code
The returned data is 00 01 C2 00, which is converted to decimal as 115200. The baud rate of RS232 is 115200.

2. 0x05 function code
This function code is used to write the controller relay
Usage example: Write FF 00 to the address 0000 relay to start the controller X-axis relative motion forward, and write 00 00 to start the control X-axis relative motion stop.
send:
01 05 00 00 FF 00 8C 3A

Slave address function code address command check code
return:
01 05 00 00 FF 00 8C 3A

3. 0x06 Function code
This function code is used to write the value of the controller register. Only one register value can be written at a time.
A 32-bit value needs to be written twice or the function code 0x10 is used.
Example of use:
Modify the controller single-axis motion X moving distance to 500 (floating point number), the address is 0x0088. The corresponding hexadecimal is: 0x43FA00. Two instructions need to be sent
send:
01 06 00 88 43 FA b8 93
take over:
01 06 00 88 43 FA b8 93

Send again:

01 06 00 89 00 00 58 20

take over:

01 06 00 89 00 00 58 20
The consistency between receiving and sending indicates that the data setting is successful.

4. 0x10 Function code
This function code is used to write multiple registers at one time. For 32-bit data, the setting can be completed by sending the data once.

Example of use:

Reference Address Form:

| Single axis motion X moving distance | 136 | 0x0088 |
| --- | --- | --- |
| Single axis moving X speed | 138 | 0x008A |
| Single axis movement X acceleration | 140 | 0x008C |
| Single axis movement X deceleration | 142 | 0x008E |

Modify the single-axis motion X moving distance to 100 (floating point number), the corresponding hexadecimal is 0X42C80000

Modify the single-axis movement X speed to 200 (floating point number), the corresponding hexadecimal is 0X43480000

Modify the single-axis movement X acceleration to 400 (floating point number), the corresponding hexadecimal is 0X43C80000

Modify the single-axis movement X deceleration to 400 (floating point number), the corresponding hexadecimal is 0X43C80000
send:
01 10 00 88 00 08 10 42 C8 00 00 43 48 00 00 43 C8 00 00 43 C8 00 00 7C 94
Slave address function code starting address digits number of bytes write data check code
take over:
01 10 00 88 00 08 41 E5

5. Single-axis motion control process:
1) Confirm that the controller lead and subdivision parameters are consistent with those of the module and driver.
2) Set the movement speed.
3) Set the acceleration.
4) Set the deceleration to.
5) Set the moving distance.
6) Start (relative/absolute) motion, or other motion.

Relay address area.

| Address Description | Decimal address | Hexadecimal address | default value | Remark |
| --- | --- | --- | --- | --- |
| X-axis forward | 0 | 0x0000 |  | Write 0XFF00 to start, write 0X00 to stop |
| Y axis forward | 1 | 0x0001 |  | Same as above |
| Z axis forward | 2 | 0x0002 |  | Same as above |
| X1 axis forward | 3 | 0x0003 |  | Same as above |
| Y1 axis forward | 4 | 0x0004 |  | Same as above |
| Z1 axis forward | 5 | 0x0005 |  | Same as above |
| X-axis inversion | 6 | 0x0006 |  | Same as above |
| Y-axis inverted | 7 | 0x0007 |  | Same as above |
| Z-axis inversion | 8 | 0x0008 |  | Same as above |
| X1 axis inversion | 9 | 0x0009 |  | Same as above |
| Y1 axis inverted | 10 | 0x000A |  | Same as above |
| Z1 axis reverse | 11 | 0x000B |  | Same as above |
| X Absolute Motion | 12 | 0x000C |  | Same as above |
| Y Absolute Motion | 13 | 0x000D |  | Same as above |
| Z Absolute Motion | 14 | 0x000E |  | Same as above |
| X1 Absolute Sports | 15 | 0x000F |  | Same as above |
| Y1 Absolute Sports | 16 | 0x0010 |  | Same as above |
| Z1 Absolute Sports | 17 | 0x0011 |  | Same as above |
| X-Pause | 18 | 0x0012 |  | Write 0XFF00 to start |
| Y Pause | 19 | 0x0013 |  | Same as above |
| Z Pause | 20 | 0x0014 |  | Same as above |
| X1 Suspension | twenty one | 0x0015 |  | Same as above |
| Y1 Pause | twenty two | 0x0016 |  | Same as above |
| Z1 Suspension | twenty three | 0x0017 |  | Same as above |
| XRecovery | twenty four | 0x0018 |  | Same as above |
| YRecover | 25 | 0x0019 |  | Same as above |
| Z Recovery | 26 | 0x001A |  | Same as above |
| X1 Recovery | 27 | 0x001B |  | Same as above |
| Y1 Recovery | 28 | 0x001C |  | Same as above |
| Z1 Recovery | 29 | 0x001D |  | Same as above |
| XStop | 30 | 0x001E |  | Same as above |
| Y Stop | 31 | 0x001F |  | Same as above |
| Z Stop | 32 | 0x0020 |  | Same as above |
| X1 Stop | 33 | 0x0021 |  | Same as above |
| Y1 stop | 34 | 0x0022 |  | Same as above |
| Z1 Stop | 35 | 0x0023 |  | Same as above |
| X starts to return to zero | 36 | 0x0024 |  | Write 0XFF00 to start, write 0X00 to stop |
| Y starts to return to zero | 37 | 0x0025 |  | Same as above |
| Z starts to return to zero | 38 | 0x0026 |  | Same as above |
| X1 starts to return to zero | 39 | 0x0027 |  | Same as above |
| Y1 starts to return to zero | 40 | 0x0028 |  | Same as above |
| Z1 starts to return to zero | 41 | 0x0029 |  | Same as above |
| X stops returning to zero | 42 | 0x002A |  | Write 0XFF00 to start |
| Y stop return to zero | 43 | 0x002B |  | Same as above |
| Z stop return to zero | 44 | 0x002C |  | Same as above |
| X1 stops returning to zero | 45 | 0x002D |  | Same as above |
| Y1 stops returning to zero | 46 | 0x002E |  | Same as above |
| Z1 stops returning to zero | 47 | 0x002F |  | Same as above |
| Start two-axis linear interpolation | 48 | 0x0030 |  | Write 0XFF00 to start |
| Start three-axis linear interpolation | 49 | 0x0031 |  | Same as above |
| Start two-axis circular interpolation | 50 | 0x0032 |  | Same as above |
| Start two-axis reverse circular interpolation | 51 | 0x0033 |  | Same as above |
| Pause interpolation | 52 | 0x0034 |  | Same as above |
| Restore interpolation | 53 | 0x0035 |  | Same as above |
| Stop interpolation | 54 | 0x0036 |  | Same as above |
| Set output port 0 | 55 | 0x0037 | Default Normally Open | Write 0XFF00 to disconnect, write 0x0000 to close |
| Set output port 1 | 56 | 0x0038 | Default Normally Open | Same as above |
| Set output port 2 | 57 | 0x0039 | Default Normally Open | Same as above |
| Set output port 3 | 58 | 0x003A | Default Normally Open | Same as above |
| Set output port 4 | 59 | 0x003B | Default Normally Open | Same as above |
| Set output port 5 | 60 | 0x003C | Default Normally Open | Same as above |
| Set output port 6 | 61 | 0x003D | Default Normally Open | Same as above |
| Set output port 7 | 62 | 0x003E | Default Normally Open | Same as above |

Register address area

By default, one parameter of FMC006GE31 uses two 16-bit registers. Therefore, when using the 03 function code to read the parameter, two registers need to be read. The value is composed of two registers. The following table lists the starting register addresses of the parameters.

Addresses 0x0000-0x0086 are read-only addresses.

| Firmware number | 80 | 0x0050 | 1001 |  |
| --- | --- | --- | --- | --- |
| Serial Number | 82 | 0x0052 | 000 | Unused |
| Device Status | 84 | 0x0054 | 01 | 01 Normal, 02 Automatic operation |
| X-axis status | 86 | 0x0056 | 0x0808 | Bitwise representation: 0X0000 power on, 0X0001 run, 0X0002 pause, 0X0004 resume, 0X0008 stop, 0X0010 negative limit, 0X0020 positive limit, 0X0040 return to zero completed, 0X0080 return to zero, 0X0100 automatic operation, 0X0200 no negative limit, 0X0400 no positive limit, 0X0800 no return to zero, 0X1000 return to zero timeout |
| Y-axis status | 88 | 0x0058 | Same as above | Same as above |
| Z-axis status | 90 | 0x005A | Same as above | Same as above |
| X1 axis status | 92 | 0x005C | Same as above | Same as above |
| Y1 axis status | 94 | 0x005E | Same as above | Same as above |
| Z1 axis status | 96 | 0x0060 | Same as above | Same as above |
| XCurrent Location | 98 | 0x0062 |  | Hexadecimal representation of floating point numbers |
| YCurrent location | 100 | 0x0064 |  | Same as above |
| ZCurrent Location | 102 | 0x0066 |  | Same as above |
| X1 Current Location | 104 | 0x0068 |  | Same as above |
| Y1 Current location | 106 | 0x006A |  | Same as above |
| Z1 Current Location | 108 | 0x006C |  | Same as above |
| X Current encoder position | 110 | 0x006E |  | Same as above |
| Y Current encoder position | 112 | 0x0070 |  | Same as above |
| Z Current encoder position | 114 | 0x0072 |  | Same as above |
| X1 Current encoder position | 116 | 0x0074 |  | Same as above |
| Y1 current encoder position | 118 | 0x0076 |  | Same as above |
| Z1 current encoder position | 120 | 0x0078 |  | Same as above |
| X Current speed | 122 | 0x007A |  | Same as above |
| Y Current speed | 124 | 0x007C |  | Same as above |
| ZCurrent speed | 126 | 0x007E |  | Same as above |
| X1 Current Speed | 128 | 0x0080 |  | Same as above |
| Y1 current speed | 130 | 0x0082 |  | Same as above |
| Z1 current speed | 132 | 0x0084 |  | Same as above |
| IO Status | 134 | 0x0086 |  | 7: Bit 0 indicates the input IO8-IO1 status, bit 1 indicates high level, bit 0 indicates low level. 15: Bit 8 indicates the output IO8-IO1 status, bit 1 indicates disconnection, bit 0 indicates closing. 21: Bit 16 indicates the positive limit of axis 6-axis 1, bit 1 indicates limit, bit 0 indicates no limit. 27: Bit 22 indicates the negative limit of axis 6-axis 1, bit 1 indicates limit, bit 0 indicates no limit. |
| Single axis motion X moving distance | 136 | 0x0088 | 50 | Hexadecimal representation of floating point numbers |
| Single axis moving X speed | 138 | 0x008A | 100 | Same as above |
| Single axis movement X acceleration | 140 | 0x008C | 200 | Same as above |
| Single axis movement X deceleration | 142 | 0x008E | 200 | Same as above |
| Single axis motion Y moving distance | 144 | 0x0090 | 50 | Same as above |
| Single axis moving Y speed | 146 | 0x0092 | 100 | Same as above |
| Single axis movement Y acceleration | 148 | 0x0094 | 200 | Same as above |
| Single axis movement Y deceleration | 150 | 0x0096 | 200 | Same as above |
| Single axis motion Z moving distance | 152 | 0x0098 | 50 | Same as above |
| Single axis moving Z speed | 154 | 0x009A | 100 | Same as above |
| Single axis movement Z acceleration | 156 | 0x009C | 200 | Same as above |
| Single axis movement Z deceleration | 158 | 0x009E | 200 | Same as above |
| Single axis motion X1 moving distance | 160 | 0x00A0 | 50 | Same as above |
| Single axis movement X1 speed | 162 | 0x00A2 | 100 | Same as above |
| Single axis movement X1 acceleration | 164 | 0x00A4 | 200 | Same as above |
| Single axis movement X1 deceleration | 166 | 0x00A6 | 200 | Same as above |
| Single axis motion Y1 moving distance | 168 | 0x00A8 | 50 | Same as above |
| Single axis moving Y1 speed | 170 | 0x00AA | 100 | Same as above |
| Single axis movement Y1 acceleration | 172 | 0x00AC | 200 | Same as above |
| Single axis movement Y1 deceleration | 174 | 0x00AE | 200 | Same as above |
| Single axis motion Z1 moving distance | 176 | 0x00B0 | 50 | Same as above |
| Single axis moving Z1 speed | 178 | 0x00B2 | 100 | Same as above |
| Single axis movement Z1 acceleration | 180 | 0x00B4 | 200 | Same as above |
| Single axis movement Z1 deceleration | 182 | 0x00B6 | 200 | Same as above |
| X return to zero speed | 184 | 0x00B8 | 50 | Hexadecimal representation of floating point numbers |
| X return to zero acceleration and deceleration | 186 | 0x00BA | 200 | Same as above |
| X return to zero drop distance | 188 | 0x00BC | 5 | Same as above |
| X return to zero direction | 190 | 0x00BE | 02 | 01 positive limit is used as the origin limit to return to zero, 02 negative limit is used as the origin limit to return to zero |
| Y return to zero speed | 192 | 0x00C0 | 50 | Hexadecimal representation of floating point numbers |
| Y return to zero acceleration | 194 | 0x00C2 | 200 | Same as above |
| Y return to zero drop distance | 196 | 0x00C4 | 5 | Same as above |
| Y return to zero direction | 198 | 0x00C6 | 02 | Same as X |
| Z return speed | 200 | 0x00C8 | 50 | Hexadecimal representation of floating point numbers |
| Z return to zero acceleration | 202 | 0x00CA | 200 | Same as above |
| Z return to zero drop distance | 204 | 0x00CC | 5 | Same as above |
| Z return to zero direction | 206 | 0x00CE | 02 | Same as X |
| X1 return to zero speed | 208 | 0x00D0 | 50 | Hexadecimal representation of floating point numbers |
| X1 zero return acceleration | 210 | 0x00D2 | 200 | Same as above |
| X1 zero return distance | 212 | 0x00D4 | 5 | Same as above |
| X1 return to zero direction | 214 | 0x00D6 | 02 | 01 positive limit is used as the origin limit to return to zero, 02 negative limit is used as the origin limit to return to zero |
| Y1 zero return speed | 216 | 0x00D8 | 50 | Hexadecimal representation of floating point numbers |
| Y1 zero return acceleration | 218 | 0x00DA | 200 | Same as above |
| Y1 zero return distance | 220 | 0x00DC | 5 | Same as above |
| Y1 return to zero direction | 222 | 0x00DE | 02 | Same as X |
| Z1 return to zero speed | 224 | 0x00E0 | 50 | Hexadecimal representation of floating point numbers |
| Z1 return to zero acceleration | 226 | 0x00E2 | 200 | Same as above |
| Z1 return to zero drop distance | 228 | 0x00E4 | 5 | Same as above |
| Z1 return to zero direction | 230 | 0x00E6 | 02 | Same as X |
| Two-axis linear interpolation axis | 232 | 0x00E8 | 0 | Indicates bit6-bit0 by bit, such as: 0X0003 represents the YX axis, 0X0021 represents the Z1, X axis |
| Two-axis linear interpolation X position | 234 | 0x00EA | 0 | Hexadecimal floating point numbers |
| Two-axis linear interpolation Y position | 236 | 0x00EC | 0 | Same as above |
| Two-axis linear interpolation speed | 238 | 0x00EE | 0 | Same as above |
| Two-axis linear interpolation acceleration | 240 | 0x00F0 | 0 | Same as above |
| Two-axis linear interpolation deceleration | 242 | 0x00F2 | 0 | Same as above |
| Three-axis linear interpolation | 244 | 0x00F4 | 0 | Indicates bit6-bit0 by bit, such as: 0X0007 represents ZYX axis, 0X0038 represents Z1, Y1, X1 axis |
| Three-axis linear interpolation X position | 246 | 0x00F6 | 0 | Hexadecimal floating point numbers |
| Three-axis linear interpolation Y position | 248 | 0x00F8 | 0 | Same as above |
| Three-axis linear interpolation Z position | 250 | 0x00FA | 0 | Same as above |
| Three-axis linear interpolation speed | 252 | 0x00FC | 0 | Same as above |
| Three-axis linear interpolation acceleration | 254 | 0x00FE | 0 | Same as above |
| Three-axis linear interpolation deceleration | 256 | 0x0100 | 0 | Same as above |
| Two-axis circular interpolation axis number | 258 | 0x0102 | 0 | Indicates bit6-bit0 by bit, such as: 0X0003 represents the YX axis, 0X0021 represents the Z1, X axis |
| Two-axis circular interpolation X position | 260 | 0x0104 | 0 | Hexadecimal floating point numbers |
| Two-axis circular interpolation Y position | 262 | 0x0106 |  | Same as above |
| Two-axis arc interpolation center X | 264 | 0x0108 |  | Same as above |
| Two-axis circular interpolation circle center Y | 266 | 0x010A |  | Same as above |
| Two-axis circular interpolation speed | 268 | 0x010C |  | Same as above |
| Two-axis circular interpolation acceleration | 270 | 0x010E |  | Same as above |
| Two-axis circular interpolation deceleration | 272 | 0x0110 |  | Same as above |
| Device ID | 274 | 0x0112 | 1 | Integer |
| 232 baud rate | 276 | 0x0114 | 115200 | Integer |
| 485 baud rate | 278 | 0x0116 | 115200 | Integer |
| CAN baud rate | 280 | 0x0118 | 1000000 | Integer |
| IP address | 282 | 0x011A | 192.168.0.30 | Integer |
| Port Number | 284 | 0x011C | 8088 | Integer |
| X Lead | 286 | 0x011E | 10 | Integer |
| Y Lead | 288 | 0x0120 | 10 | Integer |
| Z Lead | 290 | 0x0122 | 10 | Integer |
| X1 Lead | 292 | 0x0124 | 10 | Integer |
| Y1 Lead | 294 | 0x0126 | 10 | Integer |
| Z1 lead | 296 | 0x0128 | 10 | Integer |
| X Segment | 298 | 0x012A | 5000 | Integer |
| Y Segment | 300 | 0x012C | 5000 | Integer |
| Z-Subdivision | 302 | 0x012E | 5000 | Integer |
| X1 Segmentation | 304 | 0x0130 | 5000 | Integer |
| Y1 Segment | 306 | 0x0132 | 5000 | Integer |
| Z1 Segment | 308 | 0x0134 | 5000 | Integer |
| X encoder resolution | 310 | 0x0136 | 1000 | Integer |
| Y encoder resolution | 312 | 0x0138 | 1000 | Integer |
| Z encoder resolution | 314 | 0x013A | 1000 | Integer |
| X1 encoder resolution | 316 | 0x013C | 1000 | Integer |
| Y1 encoder resolution | 318 | 0x013E | 1000 | Integer |
| Z1 encoder resolution | 320 | 0x0140 | 1000 | Integer |
| X positive limit | 322 | 0x0142 | 200 | Integer |
| Y positive limit | 324 | 0x0144 | 200 | Integer |
| Z positive limit | 326 | 0x0146 | 200 | Integer |
| X negative limit | 328 | 0x0148 | 200 | Integer |
| Y negative limit | 330 | 0x014A | 200 | Integer |
| Z negative limit | 332 | 0x014C | 200 | Integer |
| X1 positive limit | 334 | 0x014E | 200 | Integer |
| Y1 positive limit | 336 | 0x0150 | 200 | Integer |
| Z1 positive limit | 338 | 0x0152 | 200 | Integer |
| X1 Negative Limit | 340 | 0x0154 | 200 | Integer |
| Y1 negative limit | 342 | 0x0156 | 200 | Integer |
| Z1 negative limit | 344 | 0x0158 | 200 | Integer |
| X return to zero timeout | 346 | 0x015A | 10000 | Integer |
| Y return to zero timeout | 348 | 0x015C | 10000 | Integer |
| Z return to zero timeout | 350 | 0x015E | 10000 | Integer |
| X1 return to zero timeout | 352 | 0x0160 | 10000 | Integer |
| Y1 return to zero timeout | 354 | 0X162 | 10000 | Integer |
| Z1 return to zero timeout | 356 | 0X0164 | 10000 | Integer |