Translated from Chinese (Simplified) to English - www.onlinedoctranslator.com

FMC006GE31 automatic control instruction table

Functional Description

The FMC006GE31 controller supports running script files. The script commands are defined by our company. The script runs from the first line to the bottom. It supports command line jumps and loops, and supports nested loops. Using this instruction system, the controller can be automatically controlled. The instructions have included all the basic functions of the controller. Different functions can be realized according to different instruction combinations.

The instruction format is: instruction parameter 1 parameter 2 parameter 3 parameter 4 remarks.

Each instruction has a maximum of 4 parameters and a minimum of 0 parameters. Comments are not downloaded to the controller.

See the table below for specific instructions. The speed parameter unit in the table is mm/s, and the acceleration and deceleration units are mm/s².

| instruction | Parameter 1 | Parameter 2 | Parameter 3 | Parameter 4 | Remark |
| --- | --- | --- | --- | --- | --- |
| Set single axis motion parameters | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | speed | Acceleration | Deceleration |  |
| Set the return to zero motion parameters | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Zero return speed | Zero return acceleration and deceleration | Return to zero direction (1, 2) corresponds to forward and reverse return to zero, 3 current position directly returns to zero |  |
| Set linear interpolation parameters | speed | Acceleration | Deceleration |  |  |
| Setting circular interpolation parameters | Bitwise representation: For example, 3 represents the YX axis, and 33 represents the Z1X axis. | speed | Acceleration | Deceleration |  |
| Start single-axis relative motion | By bit, multiple axes can be started at the same time. For example, 3 means YX axis, and YX axis can be started at the same time. 7 means ZYX, and ZYX axis can be started at the same time. ZYX and Z1Y1Z1 axis cannot be started at the same time. For example, 56 means Z1Y1Z1 axis. | Start the X-axis movement distance of the three axes ZYX or start the X1-axis movement of the three axes Z1Y1X1 | Start the Y-axis movement distance of the three axes ZYX or start the Y1-axis movement of the three axes Z1Y1X1 | Start the Z axis movement distance of the three axes ZYX or start the Z1 axis movement of the three axes Z1Y1X1 |  |
| Start single-axis absolute motion | Same as above | Same as above | Same as above | Same as above |  |
| Start single axis return to zero motion | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | The origin drop distance, if it is 0, it will not be far away from the limit switch after returning to zero | none | none |  |
| Start two-axis linear interpolation (starting from the current point) | Bitwise representation: For example, 3 represents the YX axis, and 33 represents the Z1X axis. | X target position | Y target position |  |  |
| Start three-axis linear interpolation (with the current point as the starting point) | Bitwise representation: For example, 7 represents the ZYX axis, and 56 represents the Z1Y1X1 axis. | X target position | Y target position | Z target position |  |
| Start two-axis circular interpolation | X target position | Y target position | Center X position | Y position of the center of the circle |  |
| Start two-axis reverse circular interpolation | X target position | Y target position | Center X position | Y position of the center of the circle |  |
| Pause operation | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | none |  |
| Resume Operation | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | none |  |
| Stop running | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | Axis numbers (0, 1, 2, 3, 4, 5) correspond to X, Y, Z, X1, Y1, Z1 | none |  |
| Waiting for axis operation to complete | Wait for all axis movements to complete, no parameters | none | none | none |  |
| Delayed waiting | Waiting time (ms) | none | none | none |  |
| Long wait | none | none | none | none |  |
| Jump to | The line number of the instruction to jump to | none | none | none |  |
| cycle | The line number of the instruction to jump to | Number of jumps | none | none |  |
| Local input jump | Local IO numbers 0:7 are used for IO1-IO8 respectively | Trigger event (0, 1) | Target row | none |  |
| Local input jumper switch | Local IO numbers 0:7 are used for IO1-IO8 respectively | 0: open, 1: close |  |  |  |
| Exit program execution | none | none | none | none |  |
| Local output port operation | Local IO numbers 0:7 correspond to OUT1-OUT8 respectively | Output status (0, 1) | none | none |  |
| Wait for zero return to complete | Wait for all axes to return to zero, no parameters | none | none | none |  |

Instruction Detail

The basic operation of the script is: setting certain operation mode parameters and then starting a certain operation mode.

1. Set single-axis motion parameters

This instruction contains 4 parameters, which are used to set the operating parameters of single-axis motion, such as axis number, speed, acceleration, deceleration, etc. The unit is mm/s. The parameters set by this instruction will not be saved after power failure, so this instruction must be used to reset them before each execution of the command.

The axis number parameter is one of the fixed values ​​0, 1, 2, 3, 4, and 5, representing the X, Y, Z, X1, Y1, and Z1 axes respectively. Only one axis can be set in one instruction.

2. Set the return to zero motion parameters

This command contains 4 parameters

Parameter 1: axis number (0, 1, 2, 3, 4, 5 correspond to X, Y, Z, X1, Y1, Z1)

Parameter 2: Return to zero speed (mm/s)

Parameter 3: Acceleration and deceleration when returning to zero (mm/s²)

Parameter 4: Return to zero direction (1, 2) corresponds to positive limit return to zero, negative limit return to zero

3. Set linear interpolation parameters

This command contains 3 parameters, which are used for both two-axis linear interpolation and three-axis linear interpolation.

Parameter 1: Linear interpolation speed (mm/s)

Parameter 2: Linear interpolation acceleration (mm/s²)

Parameter 3: Linear interpolation deceleration (mm/s²)

4. Set circular interpolation parameters

This command contains 4 parameters

Parameter 1: Bitwise representation: For example, 3 represents the YX axis. 33 represents the Z1X axis. Any combination of two axes can be made in the six axes.

Parameter 2: Circular interpolation speed

Parameter 3: Circular interpolation acceleration

Parameter 4: Circular interpolation deceleration

5. Start single-axis relative motion

This command contains 4 parameters, and can start 1-3 axes at the same time. The filled coordinates are all relative positions, so it is a relative motion. However, due to the limited parameter setting position, the lower three-digit Z, Y, X axis and the upper three-digit Z1, Y1, X1 axis cannot be started at the same time. They need to be started separately.

Parameter 1: axis number, 3, 5, 6 correspond to XY, XZ, YZ axis respectively, this axis number is composed of 1, 2, 4. In bitwise representation, multiple axes can be started at the same time, such as: 3 represents YX axis, YX two axes can be started at the same time. 7 represents ZYX, ZYX three axes can be started at the same time. Similarly, multiple axes with the highest three digits can be started at the same time, such as: 56 represents Z1Y1Z1 three axes, which can be started at the same time.

Parameter 2: The lower three bits of X-axis movement relative to the current point or the upper three bits of X1-axis movement relative to the current point

Parameter 3: The lower three bits of Y axis movement relative to the current point or the upper three bits of Y1 axis movement relative to the current point

Parameter 4: The lower three bits of the Z axis movement relative to the current point or the upper three bits of the Z1 axis movement relative to the current point

6. Start single-axis absolute motion

This command contains 4 parameters, and can start 1-3 axes at the same time. The filled coordinates are all absolute positions, so it is an absolute motion. However, due to the limited parameter setting position, the lower three-digit Z, Y, X axis and the upper three-digit Z1, Y1, X1 axis cannot be started at the same time. They need to be started separately.

Parameter 1: axis number, 3, 5, 6 correspond to XY, XZ, YZ axis respectively, this axis number is composed of 1, 2, 4. In bitwise representation, multiple axes can be started at the same time, such as: 3 represents YX axis, YX two axes can be started at the same time. 7 represents ZYX, ZYX three axes can be started at the same time. Similarly, multiple axes with the highest three digits can be started at the same time, such as: 56 represents Z1Y1Z1 three axes, which can be started at the same time.

Parameter 2: The lower three bits of X-axis movement relative to the current point or the upper three bits of X1-axis movement relative to the current point

Parameter 3: The lower three bits of Y axis movement relative to the current point or the upper three bits of Y1 axis movement relative to the current point

Parameter 4: The lower three bits of the Z axis movement relative to the current point or the upper three bits of the Z1 axis movement relative to the current point

7. Start single-axis return to zero motion

This command contains 2 parameters, and can only start the zero return of one axis at a time.

Parameter 1: axis number (0, 1, 2, 3, 4, 5 correspond to X, Y, Z, X1, Y1, Z1 axis respectively)

Parameter 2: The distance of the limit switch after returning to zero, which can be greater than or equal to 0

8. Start two-axis linear interpolation

This instruction is used to start two-axis linear interpolation. It can start linear interpolation of planes such as XY, XZ, YZ, X1Y1, X1Z1, and Y1Z1. The combined axis number needs to be filled in. Since the parameter only has the target position, it is an absolute motion.

Parameter 1: axis number. 3, 5, and 6 correspond to the XY, XZ, and YZ axes respectively. This axis number is composed of 1, 2, and 4 and is represented by bits: for example, 3 represents the YX axis. 33 represents the Z1X axis.

Parameter 2: Target X-axis position. This X is a virtual coordinate system and has nothing to do with the axis selected for startup.

Parameter 3: Target Y axis position. This Y is a virtual coordinate system and has nothing to do with the axis selected for startup.

9. Start three-axis linear interpolation

This instruction contains 3 parameters and is used to start three-axis linear interpolation. The coordinate values ​​are all target positions and are absolute motion.

Parameter 1: Bitwise representation: 7 represents the ZYX axis. 56 represents the Z1Y1X1 axis.

Parameter 2: Target X position

Parameter 3: Target Y position

Parameter 4: Target Z position

10. Start two-axis circular interpolation

This instruction contains 4 parameters. The axis number to be started is set in the setting parameters. This instruction is clockwise circular interpolation.

Parameter 1: Target X position

Parameter 2: Target Y position

Parameter 3: X position of the center of the circle

Parameter 4: Y position of the center of the circle

11. Start two-axis reverse circular interpolation

This command is the same as above, except that it is counterclockwise arc interpolation and the parameters are the same as above.

12. Pause operation

Parameter 1: axis number 0-5, corresponding to X-Z1 axis

Parameter 2: Axis number 0-5, corresponding to X-Z1 axis

Parameter 3: axis number 0-5, corresponding to X-Z1 axis

13. Resume operation

Parameter 1: axis number 0-5, corresponding to X-Z1 axis

Parameter 2: Axis number 0-5, corresponding to X-Z1 axis

Parameter 3: axis number 0-5, corresponding to X-Z1 axis

14. Stop operation

Parameter 1: axis number 0-5, corresponding to X-Z1 axis

Parameter 2: Axis number 0-5, corresponding to X-Z1 axis

Parameter 3: axis number 0-5, corresponding to X-Z1 axis

15. Wait for the axis to complete operation

This command has no parameters and waits for all axis movements to complete. If the axis is in the running state, the script program will wait at the line where this command is located until the axis is completed before running the next line.

Parameter 1: None

Parameter 2: None

Parameter 3: None

16. Delayed waiting

This command is used to delay the script program. The script program will wait at this line until the delay time ends. The delay time unit is ms.

Parameter 1: Delay waiting time

17. Long wait

This instruction is used to stop the current program from continuing to execute. It can be used in conjunction with the input jump instruction without parameters.

18. Jump to

This command contains 1 parameter, which is used to change the script execution order and jump to the specified line for execution.

Parameter 1: target line number (greater than 0, less than or equal to the maximum line number of the program)

19. Loop

This instruction contains 2 parameters and is used for program loop.

Parameter 1: jump target line number

Parameter 2: Number of loops

20. Local input jump

This command contains 3 parameters, which are used to detect external input. When the external input triggers the corresponding event, the script program can jump to the specified line.

Parameter 1: IO number to be detected (0:7)

Parameter 2: The event of the IO port that needs to be detected (0: rising edge event, 1: falling edge event)

Parameter 3: The line number to jump to after the event occurs

21. Exit the program

This command has no parameters and is used to exit the current script program and change the device operation mode to manual mode.

22. Local output port operation

This command contains 2 parameters, which are used to set the status of the local output port.

Parameter 1: IO number to be set (0:7 corresponds to OUT1-OUT8)

Parameter 2: The IO port status to be set (0: on, 1: off)

23. Wait for zero return to complete

This command has no parameters and waits for all axes to detect whether the zero return of each axis is completed. When each axis is in the state of zero return, this command will continue to wait. When the zero return is completed or the zero return timeout occurs, this command will execute the next operation.

Parameter 1: None

Parameter 2: None

Parameter 3: None