FMC006GE31 Function Introduction

一．Function Overview:

- FMC006GE31 is a 6-axis controller that can support six-axis mechanical motion pulse control outputs simultaneously.

- The operating voltage of the FMC00GE31 controller is 24V DC.

- The 6-axis controller supports the following motions: simultaneous single-axis relative motion of 6 axes, simultaneous single-axis absolute motion of 6 axes, 6-axis positive limit return to zero motion, 6-axis negative limit return to zero motion, linear interpolation motion of any 2 axes among 6 axes, linear interpolation motion of any 3 axes among 6 axes, positive circular interpolation motion of any 2 axes among 6 axes, and reverse circular interpolation motion of any 2 axes among 6 axes.

- The 6-axis controller supports 6 positive limit and negative limit signal input detection, which can protect the overshoot of the mechanical motion module.

- The 6-axis controller supports 8 local IO input detection, which can be used for instruction jump in automated programming (low level is valid) or ordinary IO switch detection.

- The 6-axis controller supports 8 local IO outputs, normally open output.

- Through the host computer configuration software, the 6-axis controller supports configuration sending and uploading of device parameters.

- The 6-axis controller supports automated programming through the host computer configuration software.

- The 6-axis controller supports firmware upgrades through the host computer configuration software.

- The 6-axis controller supports RS232 communication interface, RS485 communication interface, and supports modbus communication protocol based on RS232 and RS485. The 6-axis controller is a slave station of mobus and is controlled by the master station.

- The 6-axis controller supports CAN communication interface and canopen communication protocol. The 6-axis controller is a node device in canopen protocol and is controlled by the master station.

- The 6-axis controller supports the etherCAT communication interface, and supports input and output communication interfaces. The master station can realize networking through the etherCAT communication interface, control multiple 6-axis controllers, and realize axis output control in multiples of 6. In the etherCAT communication protocol, it is controlled by the master station as a slave station.

- The 6-axis controller supports TCP communication interface. As a TCP server, you can use the secondary development library to control the controller through the TCP interface.

- The 6-axis controller supports parameter reset. By long pressing the reset button, you can restore the factory settings.

- The 6-axis controller supports power-off memory function. The battery power supply can be turned on and off through a physical switch. The battery is rechargeable and has a battery charging indicator.

- The 6-axis controller supports 3 LED indicators to indicate the device operating status.

- The 6-axis controller supports 6 encoder signal input detection.

- Supported motions of the 6-axis controller：

The motion control hardware interface of the 6-axis controller is as follows:

| 5V | Connect to the driver PUL1+, DIR1+ |
| --- | --- |
| PUL1 | PWM pulse interface, connected to driver PUL1- |
| DIR1 | Direction control, connected to driver DIR1- |
| 5V | Connect to the driver PUL2+, DIR2+ |
| PUL2 | PWM pulse interface, connected to driver PUL2- |
| DIR2 | Direction control, connected to driver DIR2- |
| 5V | Connect to the driver PUL3+, DIR3+ |
| PUL3 | PWM pulse interface, connected to driver PUL3- |
| DIR3 | Direction control, connected to driver DIR3- |
| 5V | Connect to the driver PUL4+, DIR4+ |
| PUL4 | PWM pulse interface, connected to driver PUL4- |
| DIR4 | Direction control, connected to driver DIR4- |
| 5V | Connect to the driver PUL5+, DIR5+ |
| PUL5 | PWM pulse interface, connected to driver PUL5- |
| DIR5 | Direction control, connected to driver DIR5- |
| 5V | Connect to the driver PUL6+, DIR6+ |
| PUL6 | PWM pulse interface, connected to driver PUL6- |
| DIR6 | Direction control, connected to driver DIR6- |

The voltage level of pulse and direction output is 5V.

- Single-axis relative motion: FMC006GE31 supports relative motion of 6 axes at the same time, or relative motion of any combination of the 6 axes.

- Single-axis absolute motion: FMC006GE31 supports absolute motion of 6 axes at the same time, or absolute motion of any combination of the 6 axes.

- Single-axis zero return motion: FMC006GE31 supports positive limit zero return motion and negative limit zero return motion of 6 axes. The zero return timeout can be set through the host computer. Support direct zero return at the current position.

- 2-axis linear interpolation motion: FMC006GE31 supports linear interpolation motion of any 2 axes in the 6 axes. It only supports linear interpolation of any 2 axes at the same time, and does not support more than two groups of combined 2-axis linear interpolation motion within 6 axes.

- 3-axis linear interpolation motion: FMC006GE31 supports linear interpolation motion of any 3 axes in the 6 axes. It only supports linear interpolation of any 3 axes at the same time, and does not support more than two groups of combined 3-axis linear interpolation motion within 6 axes.

- 2-axis circular interpolation motion: FMC006GE31 supports positive circular interpolation motion and reverse circular interpolation motion of any 2 axes in the 6 axes. It only supports positive and reverse circular interpolation motion of any 2 axes at the same time, and does not support positive and reverse circular interpolation motion of more than two groups in the 6 axes.

- Positive and negative limit of 6-axis controller：

The hardware interface of positive and negative limit is as follows：

| L1+ | Positive limit input for 1 axis |
| --- | --- |
| L1- | Negative limit input for 1 axis |
| GNDL | Limit signal ground of 1 axis |
| L2+ | Positive limit input for 2-axis |
| L2- | Negative limit input for 2-axis |
| GNDL | Limit signal ground of 2-axis |
| L3+ | Positive limit input for 3-axis |
| L3- | Negative limit input for 3-axis |
| GNDL | Limit signal ground of 3-axis |
| L4+ | Positive limit input for 4-axis |
| L4- | Negative limit input for 4-axis |
| GNDL | Limit signal ground of 4-axis |
| L5+ | Positive limit input for 5-axis |
| L5- | Negative limit input for 5-axis |
| GNDL | Limit signal ground of 5-axis |
| L6+ | Positive limit input for 6-axis |
| L6- | Negative limit input for 6-axis |
| GNDL | Limit signal ground of 6-axis |

1. FMC006GE31 supports 6 positive and negative limit detection, and low level is effective. It can be used as a reference zero point for positive and negative return to zero movement. It can prevent accidents such as overshoot during other movements.

2. If the limit switches are not in the same power network, common ground is required, the limit signal voltage level shall not exceed 36V.

3. The positive and negative soft limit distances of the 6 axes can be configured through the host computer, which can also prevent accidents caused by overshoot.

四. IO input and output of 6-axis controller：

The IO hardware interface is as follows：

| I1 | IO Input 1 |
| --- | --- |
| I2 | IO Input 2 |
| I3 | IO Input 3 |
| I4 | IO Input 4 |
| GNDI | Input |
| I5 | IO Input 5 |
| I6 | IO Input 6 |
| I7 | IO Input 7 |
| I8 | IO Input 8 |
| GNDI | Input |

| VCCE1 | External input power positive 1 |
| --- | --- |
| GNDE1 | External input power ground 1 |
| O1 | IO output 1 |
| O2 | IO output 2 |
| O3 | IO output 3 |
| O4 | IO output 4 |
| VCCE2 | External input power positive 2 |
| GNDE2 | External input power ground 2 |
| O5 | IO output 5 |
| O6 | IO output 6 |
| O7 | IO output 7 |
| O8 | IO output 8 |

- FMC006GE31 supports 8 switch input signal detection, low level is effective. 8 input switch can be used as command jump signal for automatic programming. It can also be used as ordinary switch input detection. Both applications can exist at the same time. The maximum voltage level of the input IO port does not exceed 36V.

- If the input signal and the controller are not in the same power network, a common ground is required.

- FMC006GE31 supports 8 switch output: 8 switch normally open output, output high disconnect, output low close, can be connected to 5V, 12V, 24V and other voltage levels of relay modules, optocoupler modules, etc., the maximum voltage does not exceed 36V。

五. RS232 and RS485 communication interface：

The hardware interface of RS485 and RS232 is as follows：

| R | 120 ohms matching resistor |
| --- | --- |
| B | Connect to RS485’s B |
| A | Connect to RS485’s A |

- The 6-axis controller supports RS232 communication interface, and the hardware interface is a standard DB9 female interface. RS232 communication interface and RS485 communication interface both support modbus communication protocol. For specific modbus communication protocol data and address description, please refer to FMC006GE31-Modbus Address Definition Table.

- The 6-axis controller supports RS485 communication interface and 120 ohms resistor matching. The matching and disconnection of 120 ohms resistor can be realized through the hardware interface. It also supports modbus communication protocol with RS232. For details, please refer to FMC006GE31-Modbus Address Definition Table. In the communication protocol, it is controlled by the master station as a slave station.

- If RS485 needs to be connected to a 120 ohms resistor, use a short wire to connect the R pin externally to the B pin.

- CAN communication interface：

The hardware interface of the CAN communication interface is as follows：

| R | 120 ohms matching resistor |
| --- | --- |
| H | Connect to the high level pin of CAN bus |
| L | Connect to the low level pin of CAN bus |

- The 6-axis controller supports the CANopen communication protocol and is suitable for the field of industrial control. For specific object dictionary data definitions and instructions, please refer to FMC006GE31-Canopen Object Dictionary Table.

- The frame type uses a standard frame. The CAN master station can send and configure the data of the 6-axis controller through the data frame, send control instructions to execute mechanical motion, and query the data, configuration, and motion execution results of the six-axis controller through the remote frame. PDO transmission mode is adopted.

- The CAN communication interface also supports 120 ohms resistor matching. The matching and disconnection of 120 ohms resistors can be achieved through the hardware interface. If you need to connect to a 120 ohm matching resistor, use a short wire to connect the R pin externally to the H pin.

- The 6-axis controller is a device node of the CANopen protocol, a slave station, and is controlled by the master station. Through the CAN bus, it can realize the function of networking with other devices.

- Ethercat communication interface：

The hardware interface is as follows：

| ECATIN | Ethercat input interface |
| --- | --- |
| ECATOUT | Ethercat output interface |

- EhterCAT real-time industrial Ethernet communication has the characteristics of high speed, high data efficiency, and supports multiple device connection topologies, which is suitable for the field of industrial control. The 6-axis controller supports the ethercat communication protocol. You can use TwinCAT software to view input data points and default values, control outputs, send instructions, configure parameters, etc. As shown in the figure above, Input mapping is input data, and OUTPUT mapping is output data.

- Ethercat object dictionary data definition, check FMC006GE31-EtherCAT Object Dictionary Table for details.

- The 6-axis controller is controlled by the master station as a slave station of ethercat.

- Through the input and output ports of ethercat, the 6-axis controller can be networked and the axis control in multiples of 6 can be realized. Theoretically, the number of devices connected by ethercat can reach 65535, and the capacity of the network is almost unlimited. This greatly improves the expansion capability of the six-axis controller.

- TCP communication interface：

- The 6-axis controller supports the TCP communication interface. As a TCP server, the TCP client can access and control the controller by providing a secondary development library. For a detailed description of the secondary development library function, check FMC006GE31-Secondary Development Library Description.

- Secondary development library, providing windows-win32 version dynamic library, windows-x64 version dynamic library, ubuntu-64 bit dynamic library, arm-64 bit dynamic library, arm-32 bit dynamic library.

- The IP address and port number can be configured through the host computer. Through a switch or router, the 6-axis controller networking function can be realized in the local area network, and the control axis of the controller can be expanded by multiples of 6.

- Reset button：

- The 6-axis controller supports restoring factory settings. Pressing the reset button for more than 5 seconds can restore the factory settings.

- After restoring the factory settings, the device configuration parameters will be reset to the default parameters, and the automation programming program stored in the controller will also be deleted.

- Main and backup power：

| 24V | 24V DC input |
| --- | --- |
| GND | Power Ground |
| PE | Ground |
| ON | Backup power on |
| OFF | Backup power off |

- The 6-axis controller supports automatic switching between main and backup power. The backup power can be turned on and off by touching the switch.

- When the backup power is connected, the 6-axis controller supports the power-off memory function. When the main power is disconnected, the controller can still operate normally, memorize the current motion position, the current position recorded by the encoder, and stop mechanical motion. After the main power is restored, it automatically switches to the main power and resumes mechanical motion.

- The backup power supports charging and discharging, and automatically charges the backup power when the main power is engaged.

- LED Indicator：

| BAT | Backup power indicator |
| --- | --- |
| DATA | Data indicator light |
| ALA | Fault light |
| SYS | System operation indicator |

1．The 6-axis controller supports three colors of device operation status indicator lights, one backup power charging and discharging indicator light.

2．The Sys status indicator is green, and it will flash at 50ms intervals after the program is running normally, indicating that the controller is running normally.

- The ALA fault indicator light is red and will light up when the device fails, such as device initialization error, etc.

- DATA indicator light is yellow. It will flash when the firmware is upgrading data and will be on when the firmware is refreshed.

- The BAT backup power indicator is green. When the device has no backup power, the light will flash. When the backup power is charging, the light will be on. When the device is fully charged, the light will be off.

- 十二．Encoder interface：

| AO1+ | 1 axis A phase input positive |
| --- | --- |
| AO1- | 1 axis A phase input negative |
| BO1+ | 1 axis B phase input positive |
| BO1- | 1 axis B phase input negative |
| GNDE | 5V output power ground |
| AO2+ | 2-axis A phase input positive |
| AO2- | 2-axis A phase input negative |
| BO2+ | 2-axis B phase input positive |
| BO2- | 2-axis B phase input negative |
| 5VE | 5V output |
| AO3+ | 3-axis A phase input positive |
| AO3- | 3-axis A phase input negative |
| BO3+ | 3-axis B phase input positive |
| BO3- | 3-axis B phase input negative |
| GNDE | 5V output power ground |
| AO4+ | 4-axis A phase input positive |
| AO4- | 4-axis A phase input negative |
| BO4+ | 4-axis B phase input positive |
| BO4- | 4-axis B phase input negative |
| 5VE | 5V output |
| AO5+ | 5-axis A phase input positive |
| AO5- | 5-axis A phase input negative |
| BO5+ | 5-axis B phase input positive |
| BO5- | 5-axis B phase input negative |
| GNDE | 5V output power ground |
| AO6+ | 6-axis A phase input positive |
| AO6- | 6-axis A phase input negative |
| BO6+ | 6-axis B phase input positive |
| BO6- | 6-axis B phase input negative |
| 5VE | 5V output |

- The 6-axis controller supports 6-channel encoder input signal detection. It can detect 6-channel encoder input detection signals at the same time.

- Differential signal access, AOx+ is connected to other devices AO+, AOx- is connected to other devices AO-, BOx+ is connected to other devices BO+, BOx- is connected to other devices BO-. In order to reduce interference, it can be grounded with other devices through the GNDE pin.

- For other non-differential signal encoders, 5VE and GNDE can be used to supply 5V to the encoder. AOx+ is connected to the encoder AO+, and BOx+ is connected to the encoder BO+. AOx-, BOx-, and GNDE are connected together to share the ground.

- The encoder supports incremental encoder signal input, but does not support other types of encoder input.

- The resolution of the encoder can be configured through the host computer. Through the encoder interface, other master stations can achieve closed-loop control of motion through various communication protocols.

- Introduction of the host computer：

- The main interface is as follows, mainly including the toolbar, device list bar, main display area, print output bar, and status monitoring bar. After connecting the device, click the device in the drop-down list of the device list to open the main display area:

2. You can connect and disconnect the controller by clicking the connect button. Set the IP address and port number of the device, and click the confirm button to connect to the controller.

3. The main display area is as follows：

1）Real-time display of the current position of the 6 axes, which is the position data calculated by the internal pulse.

2）Real-time display of the current position of the 6 axes, which is the position data calculated by the encoder pulse. The accuracy of the data is related to the resolution of the encoder.

3）The 6 axis forward motion buttons. Click the current axis to move forward, release the button to stop the current axis movement.

4）The 6 axis reverse motion buttons. Click the button to move the current axis backwards, release the button to stop the current axis movement.

5）The motion control area of each axis of the 6 axes, if it is absolute motion, the target position is the absolute position, if it is relative motion, the target position is the relative motion position. You can set the target speed, target acceleration, and target deceleration.

6）The 6-axis zero return motion control area, can set the zero return speed, zero return acceleration and deceleration, and zero return direction. If you fill in 1, it means the zero return direction is positive return, if you fill in 2, it means the zero return direction is negative return, if you fill in 3, the current position will return to zero. The zero return drop distance represents the distance from the origin after zero return.

7）2-axis linear interpolation area: X-axis target position, Y-axis target position, not the actual X and Y axes. They are virtual X-axis target position and Y-axis target position after the interpolation axis is selected. You can set the interpolation speed, interpolation acceleration, and interpolation deceleration. You can select any 2 axes in the 6 axes as the interpolation motion axes. At the same time, only one group of 2 axes in the 6 axes can be interpolated.

8）3-axis linear interpolation area: Like 2-axis linear interpolation, the X-axis target position, Y-axis target position, and Z-axis target position are not the actual X, Y, and Z axes. They are virtual X-axis target positions, Y-axis target positions, and Z-axis target positions after the interpolation axis is selected. You can set the interpolation speed, interpolation acceleration, and interpolation deceleration. You can select any 3 axes in the 6 axes as the motion axes for interpolation motion. At the same time, only one group of 3-axis interpolation motion can be performed in the 6 axes.

9）2-axis circular interpolation motion: X-axis target position, Y-axis target position, are virtual X-axis target position, Y-axis target position after the interpolation axis is selected, not the actual X-axis and Y-axis target positions. You can set the center coordinates X and Y. Set the radius, interpolation speed, acceleration, deceleration, and motion direction. There can only be one set of 2-axis circular interpolation motion in the 6 axes at the same time.

10）Local input IO, output IO: IN0-IN7, low level option box is selected. OUT0-OUT7 select the option box to output low level.

11）Limit signal: positive and negative limits of the 6 axes. If the limit is triggered, the corresponding axis option box is selected. If the limit signal disappears, it is not selected. The limit signal is valid at low level.

12）Device parameter area: The device parameters can be configured to the controller through the host computer. The device parameters include device ID, RS232 baud rate, RS485 baud rate, IP address, IP port, CAN interface baud rate, CAN baud rate supports 1MBPS, 500KBPS, 250KBPS, 125KBPS.

13）The motion parameter configuration area of the 6 axes can be configured with lead, subdivision, zero return timeout, soft positive limit distance, soft negative limit distance, and encoder resolution. The encoder resolution is related to the configuration of each device. For example, if the encoder resolution of the servo drive is 10000, then the encoder resolution here is configured to be 2500. Some ordinary encoders have a resolution of 1000, so the resolution here is 1000. It depends on the specific device.

14）Control buttons for uploading and downloading device parameters, supporting uploading and downloading of device parameters.

15）The main area of automatic programming. For specific automatic programming instructions, please refer to FMC006GE31-Automatic Programming.

16）Automation programming control area. For detailed operation, please refer to FMC006GE31-Automation Programming.

17）This button can be used to upgrade the controller's firmware. After the upgrade, reconnect the device and click the Get Version Description button to see the upgraded version number.

十四. Use Case：

1. Provides a demo of the secondary development library for Windows.

2. Provides a demo of the secondary development library for Linux.

3. Provide a demo of the Canopen master station.

4. Provide demo of Modbus master station.

5. Provide a demo of the Ethercat master station.

6. The secondary development library is an access controller function library based on TCP client, providing windows 64-bit, windows 32-bit, linux-arm-64-bit, linux-arm-32-bit, Ubuntu-X86 and other versions.